Table of Contents

# TG500 Firmware Image Identity and Version Lock

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-01 |
| Version | v0.1 (draft) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 7-①: Full firmware image name & version number (locked to 25.12.x) |

## 1. Software Baseline and Version Lock Commitment

| Item | Value | Evidence |
| --- | --- | --- |
| Upstream baseline | OpenWrt 25.12 branch (frozen branch required by customer) | Aligned with the module vendor’s 25.12 support branch |
| Product versioning | TesproOS 32-2.x (distribution ID: TesproOS) | TG500 SKU build configuration |
| Kernel | Linux 6.12.y (LTS) | Build target kernel series 6.12 |
| Toolchain | gcc 14.3.0 / musl libc | TG500 SKU build configuration |
| Build target | mediatek/filogic, runtime architecture aarch64_cortex-a53 | OpenWrt target definition |
| Lock policy | Baseline frozen at 25.12.x for the contract term: only security/defect fixes cherry-picked onto the frozen release branch; no upstream base upgrade, no kernel major-version bump | Release process |

At release, a frozen release branch is cut from the development line and tagged (e.g. v32-2.3.0-TG500-R1). The tag is the single traceability anchor of the delivered version; the <rREV-HASH> segment in the image file name is the git revision and short commit hash of that tag.

## 2. TG500 Image Definition

| Item | Value |
| --- | --- |
| Device profile | TG500 device definition tespro_tg500 (TesproOS build system) |
| Device tree | mt7987a-tespro-tg500 (compatible tespro,tg500) |
| Boot media | SPI-NOR; firmware partition @0x100000, 64512 KiB (bootloader region 0x0–0xE0000 pre-flashed by the module vendor, read-only) |
| Image format | FIT (lzma kernel + external squashfs rootfs + dtb), with sysupgrade metadata |
| Produced images | …-squashfs-sysupgrade.itb (upgrade/production image), …-initramfs-recovery.itb (recovery image) |

## 3. Image Naming Scheme and Full Name

Naming scheme (OpenWrt IMG_PREFIX convention):

tesproos-<VERSION_NUMBER>-<VERSION_CODE>-<PRODUCT_TAG>-<BOARD>-<SUBTARGET>-<DEVICE>-<IMAGE_TYPE>
   │            │                │                │            │         │          │
   │            │                │                │            │         │          └ squashfs-sysupgrade.itb / initramfs-recovery.itb
   │            │                │                │            │         └ tespro_tg500
   │            │                │                │            └ mediatek-filogic
   │            │                │                └ product tag (tg500)
   │            │                └ r<REV>+<N>-<commit-hash> (git revision)
   │            └ release version number (fixed at release)
   └ distribution name

Full name example (current build-line format):

tesproos-32-2.1.0-r530+1-0eeca58f53-tg500-mediatek-filogic-tespro_tg500-squashfs-sysupgrade.itb
tesproos-32-2.1.0-r530+1-0eeca58f53-tg500-mediatek-filogic-tespro_tg500-initramfs-recovery.itb

Official deliverable images uniformly carry the tg500 product tag (as above); every release ships a sha256sums file for your verification.

## 4. Release Register (filled at release, delivered with the package)

| Release | git tag | VERSION_NUMBER | rREV-HASH | Image sha256 | Date | Change summary |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | 〈tag〉 | 〈32-2.x.y〉 | 〈rXXX-hash〉 | 〈sha256〉 | 〈date〉 | First delivery |
| … |  |  |  |  |  |  |

## 5. On-Device Version Self-Check (verifiable by customer)

cat /etc/openwrt_release     # DISTRIB_ID / DISTRIB_RELEASE / DISTRIB_REVISION
uname -r                     # kernel 6.12.y
ubus call system board       # model / system / release information

DISTRIB_REVISION must equal the <rREV-HASH> segment of the image file name, serving as the image-to-device same-origin check.