Table of Contents

# TG500 IDevID Enrollment Procedure

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-07 |
| Version | v0.1 (draft) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 9-②: enrollment process documentation |
| Hardware baseline | Infineon OPTIGA SLB9673 (I2C @0x2e, Vendor ID 0x15D1, Device ID 0x001C), device nodes /dev/tpm0, /dev/tpmrm0 |

## 0. Prerequisites

# Firmware ships kmod-tpm-tis-i2c + tpm2-tss + tpm2-tools (=y, pre-installed).
tpm2_getcap properties-fixed | grep -E "TPM2_MANUFACTURER_3|TPM2_VENT"   # expect 0x15D1
ls -l /dev/tpm*

# Required only for the OpenSSL CSR steps in §3 (TPM2 provider is an opkg package, not pre-installed):
opkg install tpm2-openssl

## 1. Locking the Hierarchy (recommended on the production line)

# Empty owner auth → set a production-controlled value (record in the MES per unit)
tpm2_changeauth -C o "" <owner-auth>

Endorsement hierarchy (EK) needs no enrollment; Owner/Platform auth must be cleared or fixed before shipment, otherwise downstream operations (evictcontrol persistence, NV write) fail.

## 2. Generating the IDevID Key Inside the TPM

# ECC P-256 restricted signing key; fixed parent, non-volatile; sensitive part never leaves the TPM
tpm2_createprimary -C e -g sha256 -G ecc256 -a "sign|restricted|fixedtpm|sensitiveorigin|noDA" \
    -c idovid.ctx --auth <idovid-auth>
tpm2_readpublic -c idovid.ctx -f pem -o idovid_pub.pem -Q

Attribute notes: fixedtpm (cannot be duplicated out), sensitiveorigin (generated inside the TPM), noDA (avoids lockout), restricted|sign (an IEEE 802.1AR compliant IDevID signing key).

## 3. Producing the CSR and Getting It Signed by the Manufacturer CA

First persist the key to its fixed handle (§4 step 1), then generate the CSR signed by the in-TPM private key through the OpenSSL TPM2 provider — the private key never leaves the TPM:

# subject must contain hardware-type/serial-number per IEEE 802.1AR, e.g.
#   serialNumber=<TPM serial + MAC>, CN=<device-serial>
openssl req -new -provider tpm2 -key 0x81000001 \
    -subj "/C=CN/O=Tespro/OU=IoT/CN=<device-serial>" -out idovid.csr

Submit idovid.csr to your own CA, or to the Tespro factory CA service (recommended to agree the PKI hierarchy and certificate profile in advance: profile OID 1.3.6.1.5.5.7.3.2, proxy 1.3.6.1.5.5.7.8.5), and obtain idovid_cert.pem.

If the TPM vendor supports a factory-baked IDevID (customized part ordering code), skip §2–§3 and read the existing certificate directly (see §6).

## 4. Persisting the IDevID (TPM NV + Persistent Handle)

# 1) Persist the key to fixed handle 0x81000001 (survives reboot, write-once)
tpm2_evictcontrol -C o -c idovid.ctx 0x81000001

# 2) Store the certificate in an owner NV index (owner auth required; define once per unit)
tpm2_nvdefine 0x01C90000 -C o -s <cert-bytes> -a "ownerread|ownerwrite"
tpm2_nvwrite 0x01C90000 -C o -o 0 --input idovid_cert.pem

NV index plan: 0x01C90000 IDevID cert (1st slot per TCG “TPM Profiles” reserved range), 0x01C90001 IDevID intermediate CA cert (optional).

## 5. Verifying the Enrollment

# Public key readable, handle exists
tpm2_readpublic -c 0x81000001
# Read the certificate back from NV and verify the chain
tpm2_nvread 0x01C90000 -C o > idovid_readback.pem
openssl verify -CAfile <ca-chain.pem> idovid_readback.pem
# Signature verification (IDevID signs an enrollment challenge)
echo -n "<nonce>" > ch.bin
tpm2_sign -c 0x81000001 -g sha256 -o ch.sig -t plain ch.bin
openssl dgst -sha256 -verify idovid_pub.pem -signature ch.sig ch.bin

## 6. Using IDevID in the Onboarding Flow (MUD / BRSKI / Custom)

For IEEE 8126 MUD or BRSKI voucher onboarding, present the IDevID certificate chain and signature with the above handle at the network onboarding stage. If the customer PKI has no manufacturer trust anchor, an EK-based identity attestation alternative is available (tpm2_evictcontrol -C e to persist an EK-based key).

## 7. Production Line Implementation Recommendations

- Wrap §1–§4 into one MES script (enroll-idovid.sh <sn> <mac> <ca-url>); per unit: enroll → verify (§5) → upload record (SN + IDevID public key fingerprint).

- Keep idovid.ctx/idovid_cert.pem in an encrypted temporary directory, wipe after production; handle persistence (§4) is write-once — a duplicate enrollment must fail fast and flag the rework queue.

- Production CA vs. field-rotation CA use separate hierarchies; keep the IDevID cert lifetime ≥ product lifetime (recommend 30 years, offline root).

- Full TPM operation reference: see the TG500 TPM configuration guide accompanying your delivered firmware.

## 8. Standards Compliance

IEEE 802.1AR (Secure Device Identity), IETF BRSKI (RFC 8995), TCG TPM 2.0 Library Specification (persistent handles 0x81000000 range / NV index owner defined range), Infineon SLB9673 datasheet.