Table of Contents

# TG500 Fuse-based Secure Boot Policy and NXP-HAB Equivalence

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-10 |
| Version | v0.1 (draft) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 4-③: fuse-based secure boot policy document (in place of the NXP HAB terminology) |
| Note | As agreed, this document uses “fuse-based secure boot policy” in place of the NXP-specific “fuse-HAB” wording |

## 1. Terminology Mapping

| Customer template term | Tespro/TG500 equivalent | Description |
| --- | --- | --- |
| NXP HAB (High Assurance Boot) | MediaTek-Gen4 secure boot chain (BootROM + FIP + eFuse) | SoC-level signature verification mechanism, NXP i.MX equivalent |
| fuse-HAB policy | Fuse-based secure boot policy | Burning the root-of-trust public key hash into eFuse fuses, irreversible |
| SRK table / SRK fuse | FIP header public-key hash (HK) written into eFuse | MediaTek uses the “ROM key hash (HK)” concept |
| HAB signed image | MTK FIP image (certificate table + entry table + signed entries) | Bootloader level |
| Chain of trust to Linux | U-Boot FIT signature → OpenWrt FIT → usign sysupgrade/packages | See trust chain in the Enablement Guide |

## 2. Fuse-based Secure Boot Policy (TG500)

- Root of trust: eFuse bits are programmed once during final production, storing the hash of the FIP header signing public key; the SoC BootROM (mask ROM) is the immutable verification anchor.

- Full chain: BootROM → BL2 → FIP entry table verification → U-Boot (FIT_SIGNATURE) → OpenWrt FIT (kernel/dtb/rootfs) → runtime sysupgrade/package signatures — every stage verified before execution, equivalent in assurance to the NXP HAB chain.

- State transitions: sample/development state (verification only, fuse unburned, test keys permitted) → production state (eFuse burned, production keys only). The transition is irreversible; a key ceremony record and dual-control are required (Enablement Guide §5).

- Failure behaviour: any failed verification halts boot; the device falls back to the signed recovery image, keeping field recovery possible without disabling the security policy.

- Auditability: verification log lines (per-stage hash check pass/fail) are collectable from boot logs; we provide an equivalence mapping table with the delivered firmware so your auditors can cross-reference.

## 3. Equivalence Statement to Your “fuse-HAB” Checklist

| fuse-HAB check item | TG500 fulfilment |
| --- | --- |
| HW fuse for secure boot enabled (production) | Planned (depends on module vendor/MTK fuse burning window) |
| Secure Boot enabled in bootloader | Available (FIT_SIGNATURE special build from the module vendor) |
| Secure Boot enabled in Linux side | Available (FIT + usign, enabled with release R1, see Enablement Guide) |
| Anti-rollback | Supported: monotonic eFuse version counters + image-version fields (policy to agree) |
| Recovery / debug interfaces locked | UART download locked after fuse burning (per MTK default policy); console per §2-4 |
| Key ownership and escrow | Tespro production keys + module vendor bootloader keys, dual escrow (per §5) |

## 4. References

- MediaTek Gen4 platform secure boot application note (via module vendor, NDA)

- U-Boot FIT signature framework documentation (public)

- OpenWrt secure boot / usign documentation (public)

- Companion Tespro documents: Secure Boot Enablement Guide, Signed Image Flow