Table of Contents

# TG500 Secure Boot Enablement Guide

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-08 |
| Version | v0.1 (draft) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 4-①: secure boot enablement guide |
| Platform | MediaTek MT7987 (Filogic) · SPI-NOR firmware-only |

## 1. Trust Chain Overview

eFuse/ROM public key → BootROM → BL2 (FIP verified) → FIP entry table verified → U-Boot (FIT_SIGNATURE self-verified)
        → OpenWrt FIT image (kernel/dtb/rootfs verified before boot) → runtime: usign-verified sysupgrade / opkg packages

| Stage | Verification mechanism | Trust anchor location | Owner |
| --- | --- | --- | --- |
| BootROM | MTK FIP format + header signature verification | BootROM + eFuse ROM key hash | MediaTek (SoC factory) |
| BL2/FIP | FIP entry table hash+signature | eFuse-embedded public key hash (BL2 stores the full cert table) | Module vendor (bootloader part) |
| U-Boot | CONFIG_FIT_SIGNATURE + embedded public key in control DTB | Public key embedded in U-Boot image | Module vendor (bootloader part) |
| OpenWrt FIT | fit-image-verify / automatic at boot | Public key embedded in sysupgrade FIT (configurable) | Tespro |
| Runtime sysupgrade | sysupgrade -T metadata check + usign | /etc/uhttpd.key etc. | Tespro |

## 2. Prerequisites

- Bootloader part (NOR 0x0–0xE0000: BL2 + FIP + U-Boot) delivered by the module vendor. For secure boot you must request the special build with CONFIG_FIT_SIGNATURE enabled and the verification public key embedded — this is a hard prerequisite.

- MTK secure-boot toolchain and provisioning procedures (Gen4_Fip_Executable / mtkconvert and eFuse burning SOP) are controlled by the module vendor/MTK. This document covers Tespro’s FIT signing part; fuse burning is an irreversible operation requiring tri-party written confirmation.

- Tespro-side signing environment (see Firmware Signing Flow document): signing server + offline HSM/ password-protected private keys + key ceremony records.

## 3. Enabling the Verification Chain Step by Step

### Step 1 — Sign OpenWrt FIT images on the Tespro side (immediately actionable)

Build with CONFIG_FIT_SIGNATURE enabled, sign kernel/dtb/rootfs with the private key via mkimage -K/-k; the device side verifies automatically before boot/install (commands in the Firmware Signing Flow doc §2). Start with verification-only without fuse burning on TG500 sample units for regression.

### Step 2 — Bootloader chain (module vendor cooperation)

The module vendor builds BL2/FIP/U-Boot in FIP format (MTK secure boot image), embeds the full certificate table; U-Boot enables FIT signature verification and includes the public key in its control DTB, verifying OpenWrt FIT with fit-image-verify.

### Step 3 — Burn the fuse root of trust (irreversible, last step)

Program the hash of the FIP header public key into SoC eFuse (one-shot); BootROM then rejects any bootloader with an incorrect signature. SOP: first validate the complete signed chain on sample units → small batch → full production. Keep the private keys and burning tools in a dual-control safe.

### Step 4 — Runtime update path enforcement

- sysupgrade accepts only FIT images with valid usign/signature (rejects tampered images);

- opkg repository GPG-restricted (usign apk/opkg signatures);

- production images disable drop to shell (or password-protected on the console + audit-logged).

## 4. Customer Acceptance Checks

# Verify the sysupgrade image signature (offline)
/usr/bin/sysupgrade -T <image>.itb           # metadata check
fit_check_sign -f <image>.itb -s <pubkey>    # FIT signature check
# On device
logread | grep -i "verify\|signature"        # verification records at boot
fw_printenv | grep -i bootverify             # boot verification switches (if defined)

| Check | Expected result |
| --- | --- |
| Boot with a tampered FIT (flip one byte) | Boot rejected / falls back to recovery image |
| sysupgrade with an unsigned image | Rejected with “Invalid metadata / signature check failed” |
| Boot logs with a legitimate image | fit-image-verify passes, hash line printed |

## 5. Key Management Requirements

- Private keys stored offline (HSM or air-gapped generation); production signing station online only during release windows; two persons + four-eyes approval.

- Key rotation plan: a backup key pair prepared before U-Boot generation, supporting a dual-key trust chain; leakage scenario → re-signing + revocation list requires a field upgrade path agreement.

- Key ceremony records (generation/public-key registration/signing logs/escrow destruction) archived for audit.