Table of Contents

# TG500 Firmware Signing Flow (Signed Image Flow)

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-09 |
| Version | v0.1 (draft) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 4-②: firmware signing flow |

## 1. Signing Scope and Objects

| Artifact | Mechanism | Signer | Verifier |
| --- | --- | --- | --- |
| Bootloader FIP (BL2/U-Boot) | MTK FIP certificate table | Module vendor (MTK toolchain) | SoC BootROM (eFuse root of trust) |
| OpenWrt sysupgrade FIT (sysupgrade.itb) | U-Boot FIT signature (RSA/SHA256) | Tespro build server | U-Boot fit-image-verify / automatic at boot |
| Recovery FIT (initramfs-recovery.itb) | same | Tespro | same |
| Package feeds (ipk/apk) | opkg usign (ed25519) | Tespro | device-side package manager |
| sysupgrade metadata | usign + SYSCONFIG_… | Tespro | sysupgrade -T |

## 2. FIT Image Signing (Tespro Production Step)

Build with signing enabled (TG500 SKU build config):

CONFIG_FIT_IMAGE_SIGNATURE=y
CONFIG_FIT_SIGN_KERNEL=y
CONFIG_FIT_EXTERNAL_SIG_ALGO_RSA4096=y   # or per key-management policy
CONFIG_FIT_SIGNATURE=y

Key generation and signing (once per release, mkimage -k embeds public keys with required property):

# 1) One-time: generate signing key pair (stored on offline media)
openssl genrsa -out keys.fit.key 4096
# 2) Sign during image build: -K specifies control dtb template, -k points to the key directory
make IMAGE_CMD_fitz="mkimage -K control.dtb -k keys/" target/install
# 3) Verify the built artifact (pre-ship check)
fit_check_sign -f openwrt-mediatek-filogic-tespro_tg500-squashfs-sysupgrade.itb -s keys.pub

Practical note: OpenWrt re-signs the FIT on every build; signing failures are mostly caused by wrong key directory permissions (mkimage requires keys/ readable) or a missing control dtb template.

## 3. Release Signing Procedure (Four-Eyes)

Build (deterministic) → Local verification (fit_check_sign + sysupgrade -T) →
Two persons verify sha256 and signature → Sign in the signing zone (usign attach,
detached .sig file) → Archive in the release repository (image + .sig + sha256sums +
manifest + SBOM + config.buildinfo) → Release approval

Artifacts delivered to the customer every release:

tesproos-…-tespro_tg500-squashfs-sysupgrade.itb   (+ .itb.sig / sha256sums entry)
tesproos-…-tespro_tg500-initramfs-recovery.itb
openwrt-…-packages-index-signature.gpg   # package feed verification chain

## 4. Device-Side Verification Path

- Boot: U-Boot (with FIT_SIGNATURE) verifies the configurations node of sysupgrade.itb against the embedded public key; failure falls back to the recovery partition / fails to a serial prompt.

- Upgrade: sysupgrade runs checkimage (usign verifies sysupgrade metadata) + FIT verification.

- Packages: opkg verifies the list signature (ed25519) with the public key in /etc/opkg/keys/.

## 5. Reproducing This Flow in Your Own Environment

For the customer’s private-signing needs, we provide: a control-dtb template, signing key generation scripts and a fit_check_sign verification guide. The public key of the customer signing key must be embedded into the U-Boot control dtb by the module vendor (U-Boot rebuild needed; plan 8–12 weeks ahead); until then, customer-signed images only pass on non-fused sample units with test keys.