Table of Contents

# TG500 IEC 61850 Application Coexistence Written Policy

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-04 |
| Version | v0.1 (draft) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 8-①: written policy; implementation spec for Item 8-② in Annex A |

## 1. Purpose and Scope

This policy defines port control and yield behaviour on a single TG500 device between the customer-side IEC 61850 application (HiTEKS ccli, holding TCP port 3782) and the Tespro-bundled IEC 61850 service (iec61850service), eliminating communication port conflicts. It applies to all TG500 firmware (TesproOS 32-2.x / OpenWrt 25.12 base).

## 2. Definitions and Current Port Usage

| Component | Process/service | Port behaviour | Remarks |
| --- | --- | --- | --- |
| HiTEKS ccli (customer application) | customer-supplied | LISTEN TCP 3782 | customer IEC 61850 stack |
| Tespro IEC61850 service | iec61850service (procd-managed, /usr/sbin/iec61850d) + iec61850-mmsd | Southbound MMS client (outbound, default TCP 102, UCI-configurable); northbound MQTT 1883 / OPC-UA 4840 (optional) | Our firmware does not listen on 3782; no inherent conflict |
| Configuration entry | /etc/config/iec61850service | start/stop gated by global.enabled | factory default enabled=0 |

Factory state: TG500 factory images do not pre-install the three IEC61850 ipks (build-only, installed on demand — see the SKU Software Package List), i.e. zero conflict by default; this policy governs runtime behaviour once they are installed and enabled.

## 3. Policy Rules

- R1 Priority: HiTEKS ccli takes precedence over Tespro iec61850service. Port 3782 always belongs to HiTEKS.

- R2 Occupancy detection: the firmware checks whether TCP 3782 (hex 0x0ECD) is in LISTEN state (parsing /proc/net/tcp and /proc/net/tcp6) at (a) boot before service start, (b) every service start request, and (c) periodically every 60 s at runtime.

- R3 Yield action: when 3782 is detected occupied — a not-yet-started iec61850service shall not start; a running instance shall stop automatically (procd instance withdrawn, no respawn); an entry is written to syslog (tag iec61850service, notice) and to UCI state iec61850service.global.coexist_state='yield'; the user configuration global.enabled is not modified.

- R4 Recovery action: when a later check finds 3782 released and global.enabled=1, the service restarts automatically, coexist_state='active', logged to syslog.

- R5 Auditability: all yield/recovery events go to syslog (logread | grep iec61850service); ubus call iec61850service status returns the current coexistence state.

- R6 Exception: this policy may be disabled (UCI global.coexist_policy='off') only upon written customer request; the requesting party then bears the consequences of conflicts.

- R7 Change control: this policy is version-frozen with the firmware; clause changes require written customer confirmation.

## 4. Responsibilities and Delivery

- Tespro software team implements rules R2–R5 (in the iec61850service start script) per Annex A, delivers them with release R1, and regression-tests them (cases: no autostart while occupied; running instance stops when port gets taken; automatic recovery after release).

- This document is the written policy for Item 8-①; the SKU package list is delivered separately (TG500-SKU-Software-Package-List).

## Annex A: Port Management Logic — Implementation Specification (Item 8-②)

Landing point: the iec61850service start script (procd init).

# Called inside start_service() before procd_open_instance
CCLI_PORT_HEX=0ECD   # 3782

port_3782_busy() {
    # /proc/net/tcp fields: sl local rem st …; st=0A means LISTEN; 3782=0x0ECD
    grep -qi ":${CCLI_PORT_HEX} [0-9A-F:.]* 0A" /proc/net/tcp /proc/net/tcp6 2>/dev/null
}

coexist_yield() {
    logger -t iec61850service "port 3782 occupied by HiTEKS ccli, yielding (service not started)"
    uci set iec61850service.global.coexist_state='yield' 2>/dev/null
    uci commit iec61850service
}

Behaviour notes: 1. start_service(): if enabled=1 and port_3782_busy → log, set state, return 0 (no procd instance opened). 2. Runtime patrol: a lightweight procd instance or cron job (every minute) repeats the check; occupied & running → /etc/init.d/iec61850service stop (stop does not touch enabled); released & enabled=1 → start. 3. stop_service() keeps its current semantics (never modifies enabled), independent of the yield logic. 4. Test case: simulate occupancy with nc -l -p 3782 and verify all R3/R4 paths and syslog output.