Table of Contents

# TG500 SKU Software Package List

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-05 |
| Version | v0.1 (draft; the build manifest/SBOM is authoritative for formal releases) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 8-③: SKU software package list |
| Authoritative source | TG500 SKU build configuration (delivered as config.buildinfo with each firmware) |

## 1. IEC 61850 Related Packages (governed by the Coexistence Policy)

| Package | Version | Pre-installed | Role |
| --- | --- | --- | --- |
| libiec61850 | 1.6.2-1 | No (build-only ipk) | IEC 61850 protocol stack library (MMS client capability) |
| iec61850-mmsd | 1.0.0-4 | No (build-only ipk) | MMS session daemon (sole holder of libiec61850; serves iec61850d via ubus/unix socket) |
| iec61850-proto-tespro-combined | 1.0.0-7 | No (build-only ipk) | Main service process iec61850d; provides the iec61850service (procd), LuCI UI, UCI config /etc/config/iec61850service |

Note: the three packages are not pre-installed in factory images (installed on demand via opkg), hence zero conflict with HiTEKS ccli in factory state; once installed and enabled they are governed by rules R1–R7 of the Coexistence Written Policy.

## 2. TPM / Identity Security Related Packages (governed by Item 9)

| Package | Version | Pre-installed | Role |
| --- | --- | --- | --- |
| kmod-tpm | same as kernel 6.12.y | Yes | TPM core driver framework |
| kmod-tpm-tis-i2c | same as kernel 6.12.y | Yes | SLB9673 (I2C @0x2e) driver; creates /dev/tpm0, /dev/tpmrm0 |
| tpm2-tss | 4.2.0-1 | Yes | TCG TSS 2.0 software stack |
| tpm2-tools | 5.8-1 | Yes | tpm2_* CLI toolset (keys/signing/NV/EK/IDevID enrollment) |

## 3. Authoritative Form of the Full List

- Package list: every build auto-generates <image>.manifest (all ipks inside the image with versions, dependency-expanded), located beside the image (bin/targets/mediatek/filogic/).

- SBOM: <image>.bom.cdx.json (CycloneDX) in the same directory, for supply-chain security review.

- Build traceability: config.buildinfo (full .config), feeds.buildinfo, sha256sums.

- Tables 1/2 above are the summary of policy-governed packages; the manifest is authoritative in case of any discrepancy.

## 4. Version Freeze Statement

The versions above are frozen with the release branch (OpenWrt 25.12 base / TesproOS 32-2.x). Patch releases (R1, R2, …) may bump these packages only for security/defect fixes, and every bump is recorded in the Release Register (see Firmware Image Identity document, §4).