Table of Contents

# TG500 TPM IDevID Provisioning Statement

| Item | Content |
| --- | --- |
| Document No. | TG500-DELIV-06 |
| Version | v0.1 (draft; effective upon signature & seal) |
| Date | 2026-10-21 |
| Supplier | Tespro Electronics Co., Ltd. |
| Review item | Item 9-①: whether the TPM is factory-provisioned with a Manufacturer IDevID |

## 1. TPM Hardware Baseline

The TG500 ships a TPM 2.0 chip Infineon OPTIGA™ SLB9673 (I2C interface @0x2e, Vendor ID 0x15D1, Device ID 0x001C), enabled by default in firmware. The factory image pre-installs kmod-tpm/ kmod-tpm-tis-i2c (device nodes /dev/tpm0, /dev/tpmrm0), tpm2-tss 4.2.0 and tpm2-tools 5.8, supporting key generation, TPM-bound sealing and X.509 certificate chains.

## 2. Conclusion on Manufacturer IDevID Provisioning

| Item | Conclusion |
| --- | --- |
| IDevID (IEEE 802.1AR device identity) | Not factory-provisioned. Standard commercial-grade SLB9673 parts carry only the manufacturer EK (Endorsement Key) credential; an IDevID baked in at the TPM factory requires a customized part ordering code |
| EK certificate availability | Available (standard feature of Infineon commercial parts); exact CA chain per on-device readout (§4) |
| Firmware support | Fully ready: IDevID can be generated inside the TPM by our production tool and persisted per the enrollment procedure |

## 3. Roadmap Selection (please confirm one option)

| Option | Content | Lead time | Recommended |
| --- | --- | --- | --- |
| A | Tespro production-line enrollment: generate IDevID inside the TPM, bind CA signing, persist to NV — per the IDevID enrollment procedure; a sample certificate for spot-checking is provided with the first shipment | Within our production schedule | ★ |
| B | Customize the TPM part with factory-baked IDevID (Infineon OPTIGA TPM customizable ID option) | Depends on the chip supplier’s factory process and MOQ | For high-volume orders |
| C | No IDevID: EK + production-line generated device certificate is sufficient | Immediate | If the customer PKI accepts EK-based attestation |

## 4. On-Spot Verification Commands on a Sample Unit (customer inspection)

# Confirm TPM is present and readable
tpm2_getcap properties-fixed | grep -A2 -E "TPM2_MANUFACTURER_3|TPM2_PRODUCT"
# Read the EK public key (the existence of an EK cert chain can be judged via TPM2_GET_CERTIFICATE)
tpm2_readpublic -C e -g sha256 -f pem -o ek.pem

## 5. Contact

- Tespro Electronics Co., Ltd. · www.tespro.com · Info@tespro.com

| Prepared by | Approved by | Customer confirmation |
| --- | --- | --- |
| ＿＿＿＿ · Date＿＿＿＿ | ＿＿＿＿ · Date＿＿＿＿ (seal) | ＿＿＿＿ · Date＿＿＿＿ |